Electromagnets

Shirley Porter-Cesair          Charles R. Henderson

2004 So. 13th Ave.             5650 So. Wolcott

Broadview Il 60153             Chicago Il 60636

(708) 344-8917                 (312) 535-9080

Objective:

The students will make an electromagnet and determine the strength of the

electromagnet.

Materials Needed:

Small Compasses

Three feet of insulated copper wire per person

Six-volt battery

Size D batteries

Battery Holders

Pieces of Cardboard

Nails (3 inches or longer)

Paper clips, tacks, pins, or other small magnetic objects

Iron filings

Salad oil

Glass or plastic cylinder (100 ml)

Wrought iron stand

Strategy:

Activity 1

Students will work in pairs.  Each pair of students will receive a baggie

containing materials needed.  Allow the students ten or fifteen minutes to

explore and manipulate the materials.  Have one student from each group connect

their compass with wire through the holes in the cardboard.  Insert the wire

through the Fahnestock clips on both ends.  Place the wire over the compass.

What happens and why?

Concept:

The electrical current flowing through a wire will create a magnetic field.

This magnetic field causes the needle to turn at a right angle to the wire.

Reverse the connections to the battery, thus reversing the direction of the

current flow, and the needle will point in the opposite direction.

Activity 2

Using the same bag of materials, the pairs of students will begin wrapping the

wire around the nail in the same direction until a foot of wire is left at both

ends.  Insert the end wires into the Fahnstock clips.  Hold the electromagnet

over a small pile of paper clips, tacks or other small metal objects.  How many

objects does your electromagnet attract?  Take the wire off the battery

terminal, and the tacks will immediately fall off.

Concept:

The current passing through the wire produces an invisible magnetic field.  When

the current is cut off, the magnetic field disappears, then the molecules of

the iron return to their helter-skelter position and the nail loses most, but

not all, of its magnetism.

Performance Assessment:

As a result of the electromagnet mini-teach, the sixth grade students will be

able to describe the characteristics of a magnet and an electromagnet.  Both

attract metal and have magnetic fields; the electromagnet needs an electrical

current.

The students will be able to make an electromagnet with a wire, a battery, and

a nail.

The students will be able to test the strength of the electromagnet by using

more coiled wire and additional batteries and nails.

Conclusions:

In activity 1 we found that electricity can produce magnetism and magnetism can

produce electricity.  The opposite ends or poles of magnets are attracted to

each other and like ends repel.

In activity 2 we found that current through a wire produces an invisible

magnetic field.  When the current is cut off, the magnetic field disappears,

then the molecules of the iron return to their helter-skelter position and the

piece loses most, but not all, of its magnetism.